| CLUSTER | MDC: MEDIA, DESIGN, AND COMMUNICATION ARTS |
| --- | --- |
| OCCUPATION | Fashion |
| HAZARD CONTROLS | EQUIPMENT—BLIND HEM (PANT HEMMING) MACHINE |
| Engineering Controls | blind hem (pant hemming) machine with knee pedal to raise plate to insert fabric |
| - Make sure the equipment is secured to table top using a clamp. (Note: The table clamp inserts into a ridge at the centre of the base.) |  |
| Administrative Controls |  |
| - Provide adequate instruction on lab routines, job procedures, and equipment operation. - Keep the work area clean and tidy at all times. - Post safety signage. - Follow the manufacturer’s directions for operation, maintenance, and safe practices. - Inspect the machine regularly to ensure safe operation. - Ensure a clear line of sight of all students on machines. - Prevent cords from becoming trip hazards in high-traffic areas. - Tag and remove defective equipment from service. - Arrange equipment to permit safe, efficient work practices and ease of movement and clean-up. - Ensure ergonomic set-up of machine and furniture. - Plug the cord into the machine first, and then plug the cord into the socket. - Work at a safe speed. (The machine operates at a fast speed; therefore, make sure to caution beginners.) - Check the settings on the machine before you start. - Thread the machine when it is off. - Make adjustments to the machine when the machine is stopped. - Turn the handle wheel toward you. (This is unlike traditional sewing machines that are turned away from you.) - Control the lowering and lifting of the plate with the knee pedal. - Do not sew with any pressure against the knee pedal. - Ensure that correct lighting is available. - Inspect cords and plugs for fraying and exposed wires. - Use the right needle for the chosen fabric weight. - Use an industrial curve needle. - Keep food and beverages away from the machine. - Follow the school safety policy. - Provide and document ongoing safety education and training, including mandatory equipment and tool orientation. - Ensure tools are used under competent supervision. - Ensure repetitive tasks are properly paced. - Post important emergency telephone numbers and contacts. - Post lockdown procedures. - Post fire drill procedures. - Establish handwashing practices. - Ensure fire extinguishers are strategically located and marked. - Provide a first aid kit and supplies, including an eyewash station and biohazard disposal. - Correctly place and maintain smoke detectors and heat sensors. |  |
| Personal Protective Equipment (PPE) |  |
| - safety glasses or eye protection |  |